Homework 1: RISC-V

Issued: December 27 (Tue.), 2021 			         Due: Jan 02 (Mon.), 2023

What to turn in: Copy the text from your MODIFIED codes and paste it into a document. If a question asks you to plot or display something on the screen, include the plot and screen output your code generates. Submit either a *.doc or *.pdf file.

**************************************************************************************************

Problem 1 (15p): Register File

Design a RISC-V register file (RF) in Verilog. Please see the description in the lecture note

What you have to do:

- Design an RF and its test bench based on the baseline codes.

- Submit your RTL files (riscv_regfile.v and riscv_regfile_tb.v)

riscv_regfile.v

- /*Insert your code */

- reg_r4_q       <= 32'h00000000;

- reg_r5_q       <= 32'h00000000;

- reg_r6_q       <= 32'h00000000;

- reg_r7_q       <= 32'h00000000;

- /*Insert your code */

- if      (rd0_i == 5'd16) reg_r16_q <= rd0_value_i;

- if      (rd0_i == 5'd17) reg_r17_q <= rd0_value_i;

- if      (rd0_i == 5'd18) reg_r18_q <= rd0_value_i;

- if      (rd0_i == 5'd19) reg_r18_q <= rd0_value_i;

- /*Insert your code */

- 5'd10: ra0_value_r = reg_r10_q;

- 5'd11: ra0_value_r = reg_r11_q;

- 5'd12: ra0_value_r = reg_r12_q;

- 5'd13: ra0_value_r = reg_r13_q;

- /*Insert your code */

- 5'd7: rb0_value_r = reg_r7_q;

- 5'd8: rb0_value_r = reg_r8_q;

- 5'd9: rb0_value_r = reg_r9_q;

- 5'd10: rb0_value_r = reg_r10_q;

riscv_regfile_tb.v

- `timescale 1ns / 100ps

- module riscv_regfile_tb;

- wire [31:0] ra0_value_o, rb0_value_o;

- reg [4:0] ra0_i, rb0_i;

- reg [4:0] rd0_i;

- reg [31:0] rd0_value_i;

- reg wr, clk, rstn;

- parameter CLOCK_PERIOD = 10;

- // Components

- riscv_regfile u_riscv_regfile

- (  .clk_i(clk),

- .rstn_i(rstn),

- .ra0_i(ra0_i),

- .rb0_i(rb0_i),

- .rd0_i(rd0_i),

- .rd0_value_i(rd0_value_i),

- .wr(wr),

- .ra0_value_o(ra0_value_o),

- .rb0_value_o(rb0_value_o)

- );

- // CLOCK

- initial begin

- clk = 0;

- forever #5 clk = ~clk;

- end

- // Testcase

- initial

- begin

- rstn = 0;

- ra0_i = 5'b0000;        // Select R0

- rb0_i = 5'b0001;        // Select R1

- wr    = 0;

- rd0_i = 5'b0001;

- rd0_value_i = 32'h00000000;

- //#2        rstn = 0;

- #20     rstn = 1;       // Reset

- ////////// WRITE /////////

- ra0_i = 5'b0000;        // Select R0

- rb0_i = 5'b0001;        // Select R1

- #20     rd0_value_i = 32'h1234;

- #20     wr = 1;         // write to R1

- rd0_i =  5'b0001;

- #10     wr = 0;

- #20     rb0_i = 5'b0111;// Select R7

- rd0_i =  5'b0111;

- #20     rd0_value_i = 32'h5678;

- #20     wr = 1;         // write to R7

- #10     wr = 0;

- ///////////////////////////

- /////////// READ //////////

- #20     ra0_i = 5'b0100;        // Read R4

- rb0_i = 5'b0101;        // Read R5

- #20     ra0_i = 5'b0111;        // Read R7

- #20     rb0_i = 5'b0001;        // Read R1

- ////////////////////////////

- #20

- #20     rstn = 0;

- #20     rstn = 1;

- end

- endmodule

- Capture the waveform.

Problem 2 (15p): Arithmetic Logic Unit (ALU)

Design a RISC-V ALU in Verilog. Please see the description in the lecture note.

What you have to do:

- Design an ALU with FLCNZ and its test bench based on the baseline codes.

- Submit your RTL files (riscv_alu.v and riscv_alu_tb.v).

riscv_alu.v

- //-----------------------------------------------------------------

- // Flag for exception cases: Homework

- //-----------------------------------------------------------------

- // Insert your code here

- shift_right_fill_r = 16'b0;

- shift_right_1_r = 32'b0;

- shift_right_2_r = 32'b0;

- shift_right_4_r = 32'b0;

- shift_right_8_r = 32'b0;

- shift_left_1_r = 32'b0;

- shift_left_2_r = 32'b0;

- shift_left_4_r = 32'b0;

- shift_left_8_r = 32'b0;

- // Insert your code here

- if (alu_b_i[2] == 1'b1)                                     //*** shift left 4(x8) to 7(x128)

- shift_left_4_r = {shift_left_1_r[27:0],4'b0000};        //    :(alu_b_i[2:0]== 3b100 to 3b111)

- else

- shift_left_4_r = shift_left_2_r;

- if (alu_b_i[3] == 1'b1)

- shift_left_8_r = {shift_left_1_r[23:0],8'b00000000};        //*** shift left 8 to 15

- else                                                        //    :(alu_b_i[3:0]== 4'b1000 to 4'b1111 )

- shift_left_8_r = shift_left_4_r;

- // Insert your code here

- if (alu_b_i[0] == 1'b1)

- shift_right_1_r = {shift_right_fill_r[31:31], alu_a_i[31:1]};

- else

- shift_right_1_r = alu_a_i;

- if (alu_b_i[1] == 1'b1)

- shift_right_2_r = {shift_right_fill_r[31:30], shift_right_1_r[31:2]};

- else

- shift_right_2_r = shift_right_1_r;

- result_r      = (alu_a_i | alu_b_i); //Insert your code //

- result_r      = (alu_a_i ^ alu_b_i);//Insert your code //

- //Insert your code

- if (alu_a_i[31] != alu_b_i[31])

- result_r  = (alu_a_i[31] < alu_b_i[31]) ? 32'h1 : 32'h0;

- else

- result_r  = sub_res_w[31] ? 32'h1 : 32'h0;

riscv_alu_tb.v

- `timescale 1ns/1ps

- `include"riscv_defines.v"

- module riscv_alu_tb ();

- // Signals

- reg rstn;

- reg clk;

- reg [3:0] alu_op_i;

- reg [31:0] alu_a_i, alu_b_i;

- wire [31:0] alu_p_o;

- wire [4:0] flcnz;

- // ALU module

- riscv_alu

- u_riscv_alu  (

- .alu_a_i(alu_a_i),

- .alu_b_i(alu_b_i),

- .alu_op_i(alu_op_i),

- .alu_p_o(alu_p_o),

- .flcnz(flcnz)

- );

- // Clock and Reset

- parameter p=10;

- initial

- begin

- clk = 1'b0;

- forever #(p/2) clk = !clk;

- end

- initial

- begin

- rstn = 1'b0;        // negedge reset on

- #(4*p) rstn = 1'b1; // negedge reset off

- end

- // Test cases

- initial

- begin:stimuli

- // Initialization

- alu_a_i = 32'h0;

- alu_b_i = 32'h0;

- alu_op_i = `ALU_ADD;

- #(8*p)  alu_a_i = 32'h0;

- alu_b_i = 32'h0;

- alu_op_i = `ALU_SLTU;

- #(4*p)  alu_a_i = 32'h2020;

- alu_b_i = 32'h2021;

- alu_op_i = `ALU_ADD;

- #(2*p)  alu_op_i = `ALU_SUB;

- #(2*p)  alu_op_i = `ALU_AND;

- #(2*p)  alu_op_i = `ALU_OR;

- #(2*p)  alu_op_i = `ALU_XOR;

- //----------------------------------------------------------------------------------------

- // ALU operations

- //----------------------------------------------------------------------------------------

- #(8*p)

- alu_a_i = 32'h0000_2222;

- alu_b_i = 32'h0000_2222;

- alu_op_i = `ALU_SLTU;

- #(p)   $display("T=%03t ns: %h < %h : %h (Unsigned)\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- // Addition

- #(4*p)

- alu_a_i = 32'h0000_1111;

- alu_b_i = 32'h0000_1111;

- alu_op_i = `ALU_ADD;

- #(p)   $display("T=%03t ns: %h + %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- // Subtraction

- #(2*p) alu_op_i = `ALU_SUB;

- #(p)   $display("T=%03t ns: %h - %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- // Comparison

- #(2*p) alu_op_i = `ALU_SLTU;

- #(p)   $display("T=%03t ns: %h < %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- //----------------------------------------------------------------------------------------

- // Logic operations

- //----------------------------------------------------------------------------------------

- // AND

- #(4*p)

- alu_a_i = 32'h0000_1110;

- alu_b_i = 32'h0000_0111;

- alu_op_i = `ALU_AND;

- #(p)   $display("T=%03t ns: %h & %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- // OR

- #(2*p) alu_op_i = `ALU_OR;

- #(p)   $display("T=%03t ns: %h | %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- // XOR

- #(2*p) alu_op_i = `ALU_XOR;

- #(p)   $display("T=%03t ns: %h ^ %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- //----------------------------------------------------------------------------------------

- // Shift operations

- //----------------------------------------------------------------------------------------

- #(4*p)

- alu_a_i = 32'h0000_1111;

- alu_b_i = 32'h0000_0001;

- alu_op_i = `ALU_SLL;

- #(p)   $display("T=%03t ns: %h << %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- #(2*p) alu_op_i = `ALU_SRL;

- #(p)   $display("T=%03t ns: %h >> %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- #(2*p) alu_op_i = `ALU_SRA;

- #(p)   $display("T=%03t ns: %h >>> %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- #(2*p)

- alu_a_i = 32'h7FFF_FFFF;

- alu_b_i = 32'hFFFF_FFFF;

- #(2*p) alu_op_i = `ALU_SLT;

- #(p)   $display("T=%03t ns: %h < %h = %h\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- #(2*p) alu_op_i = `ALU_SLTU;

- #(p)   $display("T=%03t ns: %h < %h = %h (Unsigned)\n",$realtime/1000, alu_a_i, alu_b_i,alu_p_o);

- end

- endmodule

- Capture the waveform.

The FLCNZ flags are defined as follows:

| Flag | Opcode | Description | Remarks |
| --- | --- | --- | --- |
| C | ADD, SUB | Set C to HIGH if a carry/borrow from the most significant bit (MSB) position occurs when the operands are treated as unsigned numbers. | Arithmetic |
| F | ADD, SUB | Set F to HIGH if an overflow occurs when the operands are treated as two’s complement numbers (signed numbers). | Arithmetic |
| Z | SLT, SLTU | Perform a subtraction between Rsrc1 (a) and Rsrc2 (b) without writing back to Rdest (p) and set Z to HIGH if the result is zero | Comparison |
| L | SLT, SLTU | Perform a subtraction between Rsrc1 (a) and Rsrc2 (b) without writing back to Rdest (p) and set L to HIGH if a > b when the operands are treated as unsigned numbers. | Comparison |
| N | SLT, SLTU | Perform a subtraction between Rsrc1 (a) and Rsrc2 (b) without writing back to Rdest (p) and set the N flag if a > b when the operands are treated as signed numbers. | Comparison |

Problem 3 (20p): Memory model

Design a memory model in Verilog. Please see the description in the lecture note.

What you have to do:

- Memory model (15p)

- Design a memory and its test bench based on the baseline codes.

- Submit your RTL files (riscv_memory.v and riscv_memory_tb.v)

riscv_memory.v

- // Insert your code

- if (ird_i)

- irdata_r <= mem_r[iaddr_i[DEPTH:2]];/*Insert your code */

- end

- // Insert your code

- drdata_r <= iaddr_i;

- // Insert your code

- if (dbe_w[2] && dwr_i)              //    For HALFWORD mode, select 1 of 2 slots among 32bits

- /*Insert your code */  mem_r[daddr_i[DEPTH:2]][23:16]<= dwdata_w[23:16];     //    4'b0011            : 4'b1100

- //    lower 16 bits overwrite  higher 16 bits overwrite

- if (dbe_w[3] && dwr_i)              //    For WORD mode

- /*Insert your code */  mem_r[daddr_i[DEPTH:2]][31:24]<= dwdata_w[31:24];     //    full 32bits overwrite

riscv_memory_tb.v

- `timescale 1ns/1ps

- module riscv_memory_tb ();

- reg reset_i;

- reg clk_i;

- // Input instruction

- reg  [31:0] iaddr_i;

- reg            ird_i;

- reg [31:0] daddr_i;

- reg [31:0] dwdata_i;

- reg [1:0]   dsize_i;

- reg       drd_i;

- reg           dwr_i;

- // Outputs

- wire [31:0] irdata_o;

- wire [31:0] drdata_o;

- localparam

- SIZE_BYTE = 2'd0,

- SIZE_HALF = 2'd1,

- SIZE_WORD = 2'd2;

- // Memory module

- riscv_memory

- u_memory (

- .clk_i(clk_i),

- .reset_i(reset_i),

- .iaddr_i(iaddr_i),

- .ird_i(ird_i),

- .daddr_i(daddr_i),

- .dwdata_i(dwdata_i),

- .dsize_i(dsize_i),

- .drd_i(drd_i),

- .dwr_i(dwr_i),

- .irdata_o(irdata_o),

- .drdata_o(drdata_o)

- );

- // Clock and Reset

- parameter p=10;

- initial begin

- clk_i = 1'b0;

- forever #(p/2) clk_i = !clk_i;

- end

- // Test cases

- initial

- begin:stimuli

- reset_i = 1'b0;

- iaddr_i = 0;

- ird_i = 0;

- dsize_i=SIZE_WORD;

- #(4*p) reset_i = 1'b1;

- #(4*p)  iaddr_i = 32'h0;

- ird_i = 1'b1;

- #(p)    ird_i = 1'b0;

- $display("T=%03t ns: %h : %h\n",$realtime/1000, iaddr_i, irdata_o);

- #(4*p)  iaddr_i = 32'h4;

- ird_i = 1'b1;

- #(p)    ird_i = 1'b0;

- $display("T=%03t ns: %h : %h\n",$realtime/1000, iaddr_i, irdata_o);

- #(4*p)  iaddr_i = 32'h8;

- ird_i = 1'b1;

- #(p)    ird_i = 1'b0;

- $display("T=%03t ns: %h : %h\n",$realtime/1000, iaddr_i, irdata_o);

- #(4*p)  iaddr_i = 32'hd;

- ird_i = 1'b1;

- #(p)    ird_i = 1'b0;

- $display("T=%03t ns: %h : %h\n",$realtime/1000, iaddr_i, irdata_o);

- #(4*p)  iaddr_i = 32'h10;

- ird_i = 1'b1;

- #(p)    ird_i = 1'b0;

- $display("T=%03t ns: %h : %h\n",$realtime/1000, iaddr_i, irdata_o);

- end

- endmodule

- Capture the waveform.

- Program memory file (5p)

- Use the RISC-V simulator Venus (https://www.kvakil.me/venus/) to generate a program file.

- Modify the test bench (riscv_memory_tb.v) to simulate the memory model with the newly generated file.

- Capture the waveform and the display result that is printed out in Transcript Window.

| C code | Assembly Code |  |
| --- | --- | --- |
| int A[64]; int sum = 0; for (int i=0; i<64; i++) sum += A[i]; | addi x9, x0, 64 add x10, x0, x0 add x11, x0, x0 Loop: lw x12, 0(x9) add x10, x10, x12 addi x9, x9, 4 addi x11, x11, 1 addi x13, x0, 64 blt x11, x13, Loop | # x9=&A[0] # sum=0 # i=0 # x12=A[i] # sum+= A[i] # &A[i++] # i++ # x13=64 |

Problem 4 (15p): RISC-V Decoder

Design a RISC-V decoder in Verilog. Please see the description in the lecture note.

What you have to do:

- Design a RISC-V decoder and its test bench based on the baseline codes.

- Submit your RTL files (riscv_decoder.v and riscv_decoder_tb.v).

riscv_decoder.v

- assign ra_w = if_opcode_w[19:15];/*your code*/

- assign op_alu_reg_w = (7'b0110011 == op_w);/*your code*/

- assign op_f7_alt_w  = (7'b0100000 == f7_w);/*your code*/

- assign auipc_w  = (7'b0010111  == op_w);/*your code*/

- assign bne_w    = op_branch_w  && (3'b001 == f3_w);/*your code*/

- assign lw_w     = op_load_w    && (3'b010 == f3_w);/*your code*/

- assign sb_w     = op_store_w   && (3'b000 == f3_w);/*your code*/

- assign xori_w   = op_alu_imm_w && (3'b100 == f3_w);/*your code*/

- assign slli_w   = op_alu_imm_w && (3'b001 == f3_w) && op_f7_main_w;/*your code*/

- assign slt_w    = op_alu_reg_w && (3'b010 == f3_w) && op_f7_main_w;/*your code*/

- assign srl_w    = op_alu_reg_w && (3'b101 == f3_w) && op_f7_main_w;/*your code*/

- assign mulh_w   = op_alu_reg_w && (3'b001 == f3_w) && op_f7_mul_w;/*your code*/

- assign remu_w   = op_alu_reg_w && (3'b111 == f3_w) && op_f7_mul_w;/*your code*/

riscv_decoder_tb.v

- module riscv_decoder_tb;

- reg rst;

- reg clk;

- // Input instruction

- reg  [31:0] if_opcode_w;

- // Outputs

- wire [31:0] id_imm_w;

- wire [4:0] id_rd_index_w;

- wire [4:0] id_ra_index_w;

- wire [4:0] id_rb_index_w;

- wire [3:0] id_alu_op_w;

- wire [2:0] id_branch_w;

- wire [1:0] id_mem_size_w;

- //Flags

- wire mulh_w;

- wire mulhsu_w;

- wire div_w;

- wire rem_w;

- wire sra_w;

- wire srai_w;

- wire alu_imm_w;

- wire jal_w;

- wire load_w;

- wire store_w;

- wire lbu_w;

- wire lhu_w;

- wire jalr_w;

- wire id_illegal_w;

- riscv_decoder

- u_decoder

- (

- ./*input  [31:0]*/ if_opcode_w(if_opcode_w),

- ./*output [31:0]*/ id_imm_w(id_imm_w),

- ./*output [4:0] */id_rd_index_w(id_rd_index_w),

- ./*output [4:0] */id_ra_index_w(id_ra_index_w),

- ./*output [4:0] */id_rb_index_w(id_rb_index_w),

- ./*output [3:0] */id_alu_op_w(id_alu_op_w),

- ./*output [2:0] */id_branch_w(id_branch_w),

- ./*output [1:0] */id_mem_size_w(id_mem_size_w),

- //Flags

- ./*output */mulh_w(mulh_w),

- ./*output */mulhsu_w(mulhsu_w),

- ./*output */div_w(div_w),

- ./*output */rem_w(rem_w),

- ./*output */sra_w(sra_w),

- ./*output */srai_w(srai_w),

- ./*output */alu_imm_w(alu_imm_w),

- ./*output */jal_w(jal_w),

- ./*output */load_w(load_w),

- ./*output */store_w(store_w),

- ./*output */lbu_w(lbu_w),

- ./*output */lhu_w(lhu_w),

- ./*output */jalr_w(jalr_w),

- ./*output */id_illegal_w(id_illegal_w)

- );

- parameter p=10;

- initial

- begin

- clk = 1'b0;

- forever #(p/2) clk = !clk;

- end

- initial

- begin

- rst = 1'b0;     // negedge reset on

- if_opcode_w = 0;

- #(4*p) rst = 1'b1;  // negedge reset off

- /*

- C:

- for (int i = 0;i < 10;i++) {

- // Repeated code goes here.

- }

- Assembly:

- # t0 = 0

- li      t0, 0

- li      t2, 10

- loop_head:

- bge     t0, t2, loop_end

- # Repeated code goes here

- addi    t0, t0, 1

- loop_end:

- // Venus

- 0x00000293  addi x5 x0 0    li t0, 0

- 0x00a00393  addi x7 x0 10   li t2, 10

- 0x0072d463  bge x5 x7 8 bge t0, t2, loop_end

- 0x00128293  addi x5 x5 1    addi t0, t0, 1

- */

- #(4*p) if_opcode_w = 32'h00000293;

- #(4*p) if_opcode_w = 32'h00a00393;

- #(4*p) if_opcode_w = 32'h0072d463;

- #(4*p) if_opcode_w = 32'h00128293;

- #(4*p) if_opcode_w = 32'h0;

- end

- endmodule

- Capture the waveform.

Problem 5 (15p): Program Counter

Design a program counter in Verilog. Please see the description in the lecture note for details.

What you have to do:

- Program Counter (10p)

- Complete a program counter and test bench based on the baseline codes.

- Do simulation and capture its result, i.e., waveform and transcript window.

- Extended program counter (5p)

Compared to riscv_pc_tb1, riscv_pc_tb2 connects iaddr_i to the output of the Program Counter.

- Complete the test bench (riscv_pc_tb2.v).

riscv_pc_tb2.v

- //Insert Your code

- //{{{

- ./*input                */ird(ird_i),               // Instruction Read request

- ./*input                */branch_taken_w(branch_taken_w),   // Jump instruction

- ./*input  [PC_SIZE-1:0] */jump_addr_w(jump_addr_w),     // Jump address

- ./*output [PC_SIZE-1:0] */if_next_addr_w(if_next_addr_w)        // Next instruction

- //}}}

- //Insert Your code

- //{{{

- .iaddr_i(if_next_addr_w),

- .ird_i(ird_i),

- //}}}

- Do simulation and capture its result, i.e., waveform and transcript window.

Problem 6 (20p): Program

Design a simplified RISC-V core in Verilog. Please see the description in the lecture note.

The assembly code (Problem 2b):

What you have to do:

- Baseline core (10p)

- Use Program Counter in Problem and complete all connections for ALU, Decoder, and Register File modules.

- Do a simulation and capture its result, i.e., waveform.

- Default target address and branch enable signal for Brach-Less-Than BLT (2p)

In the baseline mode for (BLT), the target address for Brach-Less-Than (BLT) is set to 0, while the branch “enable” signal is always set to 1. Explain why r11 in Register File only has two values, 0 and 1.

Answer:

The instruction `add x11 x0 x0` will make the r11 in Register File to be 0, then in the loop `addi x11 x11 1` will make the r11 in Register File to be 1. At the loop condition judgment `blt x11, x13, Loop` will not jump to the target address which make the instruction will begin initially. Thus, After the one loop, the r11 will be initialized with 0 instead of be added 1.

- Branch instruction Brach-Less-Than (BLT) (8p)

  - Target Address for BLT (4p)

- Make code to calculate a target address for BLT.

- Do a simulation and capture its result, i.e., waveform.

  - Target enable for BLT (4p)

- Make code to generate a branch enable signal for BLT

- Do a simulation and capture its result, i.e., waveform.

Problem 7 (8p) (Optional): Bonus

- (2p) In Problem 2, a shift module can be implemented in two ways: 1) using a “case” expression; or 2) using a barrel shift approach. How many shifters with a constant shifting amount are used in both cases? Generalize the problem for given n-bit numbers a and b.

Answer: I think the barrel shift approach is more convenient and efficiency than the “case” expression. For the given n-bit numbers to shift, the “case” expression needs n-shifters and the barrel shift approach needs clog2(n)-bit numbers.

- (1p) Assume that each operation in ALU is described in a block. Draw a block diagram of ALU.

Answer:

Register

Register

ALU

- (2p) In problem 3, a slow sequential multiplier completes a 32-bit multiplication in 32 cycles. Is a computation result correct if the input is changed during computation? Modify the code to make the computation result correct even though input is changed during computation. Show the test cases and the waveform corresponding to both the baseline code and a modified code.

Answer:

Modified code:

- //Insert your code

- mul_count_r  <= mul_count_r - 5'd1;

- /*Insert your code */                               //*** counter check for state-end

- mul_res_r   <= { mul_sum_w, mul_res_r[31:1] };    //*** 64bit width register composed by previous adder result and multiplier b

- //*** uppper with adder result (or initial multiplicand a), lower with multiplier b

- //*** 1bit shift right per state change

- // Insert your code

- mul_ready_r <= 1'b1;

- // Insert your code

- mul_res_r   <= { 32'h0, id_ra_value_r};  //*** intial value with upper 32bits zero and lower 32bit multiplicand a

WaveForm:

- (3p) The following table shows C codes and their assembly codes for a program that calculates a sum of all elements of an array. Two code versions use LESS-THAN and NOT-EQUAL operations for a conditional branch in a loop. In this problem, you have to do:

- Generate a memory file for the modified assembly code using a NOT-EQUAL operation, called mem_bne.hex.

- Modify a test bench to use a new memory file (riscv_core_sim_bne_tb.v).

- `timescale 1ns / 100ps

- module riscv_core_sim_bne_tb;

- reg clk_i;

- reg reset_i;

- wire        lock;

- wire [31:0] iaddr;

- wire [31:0] irdata;

- wire        ird;

- wire [31:0] daddr;

- wire [31:0] dwdata;

- wire [31:0] drdata;

- wire  [1:0] dsize;

- wire        drd;

- wire        dwr;

- riscv_memory #(.FIRMWARE("mem_bne.hex"))

- u_riscv_memory

- (

- ./*input         */clk_i(clk_i),

- ./*input         */reset_i(reset_i),

- ./*input  [31:0] */iaddr_i(iaddr),

- ./*output [31:0] */irdata_o(irdata),

- ./*input         */ird_i(ird),

- ./*input  [31:0] */daddr_i(daddr),

- ./*input  [31:0] */dwdata_i(dwdata),

- ./*output [31:0] */drdata_o(drdata),

- ./*input   [1:0] */dsize_i(dsize),

- ./*input         */drd_i(drd),

- ./*input         */dwr_i(dwr)

- );

- riscv_core_sim

- u_riscv_core_sim

- (

- ./*input         */clk_i(clk_i),

- ./*input         */reset_i(reset_i),

- ./*output        */lock_o(lock),

- ./*output [31:0] */iaddr_o(iaddr),

- ./*input  [31:0] */irdata_i(irdata),

- ./*output        */ird_o(ird),

- ./*output [31:0] */daddr_o(daddr),

- ./*output [31:0] */dwdata_o(dwdata),

- ./*input  [31:0] */drdata_i(drdata),

- ./*output  [1:0] */dsize_o(dsize),

- ./*output        */drd_o(drd),

- ./*output        */dwr_o(dwr)

- );

- // CLOCK

- initial begin

- clk_i = 0;

- forever #5 clk_i = ~clk_i;

- end

- // Testcase

- initial

- begin

- reset_i = 1'b0;

- #20 reset_i = 1'b1; // Reset

- end

- endmodule

- Make code to compute a target address and a branch enable signal for Branch-Not-Equal (BEQ).

- // Insert your code

- //{{{

- branch_taken_w = 1'b1;

- jump_addr_w = if_pc_d + id_imm_w;

- //}}}

- Do a simulation and capture its waveform.

| C code | Assembly Code |  |
| --- | --- | --- |
| int A[64]; int sum = 0; for (int i=0; i<64; i++) sum += A[i]; | addi x9, x0, 64 add x10, x0, x0 add x11, x0, x0 Loop: lw x12, 0(x9) add x10, x10, x12 addi x9, x9, 4 addi x11, x11, 1 addi x13, x0, 64 blt x11, x13, Loop | # x9=&A[0] # sum=0 # i=0 # x12=A[i] # sum+= A[i] # &A[i++] # i++ # x13=64 |
| int A[64]; int sum = 0; for (int i=0; i≠64; i++) sum += A[i]; | addi x9, x0, 64 add x10, x0, x0 add x11, x0, x0 Loop: lw x12, 0(x9) add x10, x10, x12 addi x9, x9, 4 addi x11, x11, 1 addi x13, x0, 64 bne x11, x13, Loop | # x9=&A[0] # sum=0 # i=0 # x12=A[i] # sum+= A[i] # &A[i++] # i++ # x13=64 |